# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: “Delivery and Installation of Fabric Cutting Machine in TR”	p 1 / 7

Publication reference: BGTR0300167-LP-SUP-01

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Declaration of Conformity and CE marking for the offered equipment, or equivalent evidence of conformity with Machinery Directive 2006/42/EC, Low Voltage Directive 2014/35/EU, and EMC Directive 2014/30/EU.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Cutting & Processing Capabilities |  |  |  |
| 1.1 | Cutting Height: Min 50 mm – Max 80 mm (under vacuum) |  |  |  |
| 1.2 | Cutting Width: Min 180 cm – Max 200 cm |  |  |  |
| 1.3 | Cutting Window Length: Min 2500 mm – Max 3500 mm |  |  |  |
| 1.4 | Cutting Speed: Min 80 m/min – Max 100 m/min |  |  |  |
| 1.5 | Acceleration: Min 9 m/s2 – Max 10 m/s2 |  |  |  |
| 1.6 | Blade Vibration Speed: Min 5000 rpm – Max 6500 rpm |  |  |  |
| 2 | Vacuum System |  |  |  |
| 2.1 | Vacuum Power: Min 15 kW – Max 20 kW |  |  |  |
| 3 | Power & Electrical Characteristics |  |  |  |
| 3.1 | Average Power Consumption: Min 5 kW – Max 25 kW |  |  |  |
| 3.2 | Operating System: Windows 10 Nextgen 64-bit OS |  |  |  |
| 4 | Mechanical Dimensions & Physical Characteristics |  |  |  |
| 4.1 | Machine Length: Min 3.50 m – Max 6.00 m |  |  |  |
| 4.2 | Machine Width: Min 2.90 m – Max 3.50 m |  |  |  |
| 4.3 | Machine Weight: Min 2500 kg – Max 3500 kg |  |  |  |
| 5 | Environmental & Workplace Conditions |  |  |  |
| 5.1 | Noise Level: Min 74 dB – Max 84 dB |  |  |  |
| 5.2 | Operating Temperature: Min 0°C – Max 50°C |  |  |  |
| 6 | Software & Compatibility |  |  |  |
| 6.1 | Compatible File Formats: ISO274D, ISO6983, AAMA, DXF |  |  |  |
| 7 | Auxiliary & Post- Processing Components |  |  |  |
| 7.1 | Sorting Conveyor Length: Min 2000 mm – Max 2500 mm |  |  |  |
| 7.2 | Sorting Conveyor Width: Min 1800 mm – Max 2000 mm |  |  |  |
| 7.3 | Warm Drill – Bit Diameter: Min 1.25 mm – Max 5 mm |  |  |  |